| Project Type: | Major Land Development |  |  |
| --- | --- | --- | --- |
| Review Stage: | Pre-Application Concept Plan |  |  |
| Address: | Holley Street |  |  |
| Plat: | 57-1 | Lot: | 110 |
| Parcel Size: | ~1.3 acres | Zoning District: | CN |
| Applicant: | Scot Hallberg 17 Arnold Street, Suite 100 Wakefield, RI 02879 | Owner: | Southern Rhode Island Professional Center, LLC P.O. Box 817 Charlestown, RI 02813 |
| Current Plan Set: | Pre-Application Submission for “The Residences on Holley”, AP 57-1, Lot 110, Holley Street in South Kingstown, Rhode Island, Sheets 1 through 2, dated June 3, 2020, completed by Principe Company, Inc., P.O. Box 298, Tiverton, RI 02878. |  |  |

Property Characteristics

The subject property is an undeveloped parcel approximately 1.3 acres in size and is located in a Commercial Neighborhood (CN) Zoning District.  The parcel is located between School and Main Street and surround properties are primarily comprised of residential dwellings with some commercially used properties to south and the east.

Project Description

The applicant is proposing a major multi-household land development project to include six (6) 1,440 square foot duplex structures with a two (2) 720 square foot units each for a total of twelve (12) units.  In accordance with the inclusionary zoning requirements of the zoning ordinance, two (2) of these units (20%) will be deed restricted for low to moderate income ownership or occupancy.

Access to these units include a shared access driveway for the three (3) southernmost structures and the remaining structures to the north will be serviced by individual curb-cuts and driveways along the frontage on Holley Street.  The applicant proposes each unit to have a garage and a single external parking space dedicated to each unit and the property is proposed to be serviced by Town water and sewer service.  The applicant proposes that best management practices to mitigate stormwater runoff will be provided including roof bioretention areas and permeable/porous pavement.  Landscaping will also include plantings along the frontage and a 20-foot modified landscape screen will be installed along the westerly property limits along the residentially zoned properties abutting the subject site.

Regulatory Considerations

Purpose of the Pre-Application Review

Per the Regulations, Pre-Application meetings shall:

- Aim to encourage information sharing and discussion of project concepts among the participants:

- Include a review of the physical character of the land, any environmental or physical constraints to development; and

- Include a discussion initiated by the Planning Board regarding what form of land development may be appropriate to meet the goals and policies of the Comprehensive Plan with regard to preserving the character of the land, the natural environment and the ability of the Town to provide essential services.

Pre-application discussions are intended for the guidance of the applicant and shall not be considered approval of a project or of any of its elements.  No formal action need be taken by the Planning Board at the pre-application meeting.

Section 401 (Zoning – Dimensional Requirements)

Section 401 (Schedule of dimensional regulations) provides the maximum density allowed for multi-household land development projects.  The allowed density for the subject property is:

15,000 for first two (2) dwelling units plus 5,000 square feet per each additional dwelling unit.

At 56,527 square feet, the maximum density for the subject property is 10.31 dwelling units.

Article IV, Section H – Multi-Household Land Development Project

Section H(6) requires screening to provide a physical barrier and visual screen from adjacent properties.  Section H(7) requires a minimum front yard setback of 100 feet along public streets with prohibitions on buildings, parking lots and other utility areas from being located within this setback.  A 50’ landscaped or natural buffer zone is required to be maintained along said public street.  Section H(9) requires that pedestrian walkways be provided for the development, and that parking lots be set back from the wall of any building (except garages, that 10% of the developable area (~5,653 square feet) must be dedicated as open space, type dictated by occupants and approved by the Planning Board, and that the minimum distance between buildings shall be 50’.

A more thorough analysis of Article IV, Section H will be conducted by Staff if this project moves forward to the next stage of review.

Inclusionary Zoning

Section 502.6 (Inclusionary Zoning) of the Zoning Ordinance outlines the Inclusionary Zoning requirements and procedures for major subdivisions or land development projects that consist of six (6) or more parcels or units.  Those requirements are listed, in part, below:

E.	Base density calculation, inclusionary requirements and targeted populations. …A minimum of 20 percent of the basic maximum number of lots/units must be affordable to low and/or moderate income households as defined under G.L. 1956, §45-53, Low and Moderate Income Housing Act...To facilitate such targeting and ensure that inclusionary units are well integrated into the subdivision or land development project, the planning board shall have the authority to specify the type and location of unit(s) to be constructed, the size and scale of these units or lots in relation to the market rate units or lots in the development, establish general design parameters for the inclusionary units and determine a construction time table for construction.

F.	Zoning Incentive.  For all subdivisions, flexible design land development projects and land development projects that are subject to the provisions of this section, a zoning incentive increasing the basic maximum number of lots or dwelling units by a factor of 1.2 shall be required. Units required under the zoning incentive provisions of this section shall count toward the maximum zoning incentive permitted under subsection 502.5.E of this ordinance. The requirement for such inclusionary units shall constitute a municipal government subsidy as defined under the RI Low and Moderate Income Housing Act and this ordinance.

G.	Inclusionary units, deed restriction requirements. All inclusionary units required under this section shall meet the definition of "low and moderate income housing" as defined in G.L. 1956, § 45-53-3; provided however that units and/or lots generated under these inclusionary provisions shall remain affordable for a minimum period of 99 years. Further provided that any for-sale lots or units shall be sold or conveyed to persons or households earning up to 80 percent of the area median income (AMI) for South Kingstown as defined by the U.S. Department of Housing and Urban Development.

The developer of inclusionary units must contract with a monitoring agency that will qualify home buyers and/or renters for initial occupancy, determine pricing and resale or transfer of dwelling units in accordance with RI Housing and Mortgage Finance Corporation regulations and standards. Rhode Island Housing and the Town of South Kingstown shall be a party to any such monitoring agreement. Long-term affordability shall be guaranteed through the recording of restrictions on the sale, transfer and ownership of the unit through the appropriate legal documents and monitoring agreement.

I.	Modification of lot requirements for inclusionary units. To encourage good design and to reduce construction, land and infrastructure costs the planning board shall have the discretion to modify (or require the modification of) minimum dimensional standards for inclusionary dwelling units/lots. Such modifications shall be in accord with the standards and requirements set forth in subsection 502.5.F of this ordinance.

The zoning incentives within the Inclusionary Zoning provisions of the Ordinance allow for a total of twelve (12) units with two (2) being deed restricted affordable units.  The applicant is proposing a total of ten (10) units with two (2) deed restricted affordable units.  The applicant will need to provide a Yield Plan in accordance with Section 502.6(E) as referenced above.

Waivers Requested

The applicant has not indicated that any waivers are being requested at this time.

Required Findings

In approving this land development request, the Board must make positive findings on the following standard provisions:

- The proposed development is consistent with the comprehensive community plan and/or has satisfactorily addressed the issues where there may be inconsistencies;

- The proposed development is in compliance with the standards and provisions of the municipality's zoning ordinance;

- There will be no significant negative environmental impacts from the proposed development as shown on the final plan, with all required conditions for approval;

- The development, as proposed, will not result in the creation of individual lots with any physical constraints to development that building on those lots according to pertinent regulations and building standards would be impracticable. (See definition of Buildable lot). Lots with physical constraints to development may be created only if identified as permanent open space or permanently reserved for a public purpose on the approved, recorded plans; and

- All proposed land developments and all subdivision lots have adequate and permanent physical access to a public street. Lot frontage on a public street without physical access shall not be considered in compliance with this requirement.

The approving authority must also address each of the following general purposes of zoning:

- Providing for the orderly, thorough and expeditious review and approval of land developments and subdivisions;

- Promoting high quality and appropriate design and construction of land developments and subdivisions;

- Promoting the protection of the existing natural and built environment and the mitigation of all significant negative impacts of any proposed development on the existing environment;

- Promoting design of land developments and subdivisions which are well-integrated with the surrounding neighborhoods with regard to natural and built features, and which concentrate development in areas which can best support intensive use by reason of natural characteristics and existing infrastructure;

- Encouraging local design and improvement standards to reflect the intent of the community comprehensive plans with regard to the physical character of the various neighborhoods and districts of the municipality;

- Promoting thorough technical review of all proposed land developments and subdivisions by appropriate local officials;

- Encouraging local requirements for dedications of public land, impact mitigation, and payment-in-lieu thereof, to be based on clear documentation of needs and to be fairly applied and administered; and

- Encouraging the establishment and consistent application of procedures for local record-keeping on all matters of land development and subdivision review, approval and construction.